PROGRAM TITLE

ASSET MANAGEMENT

PROCESS TITLE

Vessel Repairs and Maintenance

- SCOPE

This policies and procedures manual establishes policies, systems, procedures and controls on vessel repairs and maintenance. All duties and responsibilities stated in this manual are not exclusive to the personnel’s designated responsibilities in this process title.

- OBJECTIVES

- To provide clear guidelines in the execution of vessel afloat and dry dock repairs, and preventive maintenance.

- To clearly define the duties and responsibilities of all personnel involved in the vessel afloat and dry dock maintenance.

- To ensure that proper documentation is established to monitor the procedures on this process title.

- PERSONNEL INVOLVED

  - Engineering Manager

    - Receives the Work Order (WO) from Operations Department by signing the “Received by” portion.

    - Advises Hull Supervisor of Dry Dock Schedule.

    - Verifies the following documents:

- Dry Dock Schedule (DDS)

- Bill of Materials (BOM)

    - Approves the following documents:

- Materials Requisition Slip (MRS)

- Purchase Requisition Form (PRF)

  - Assistant Hull Supervisor

    - Receives advise from the Hull Supervisor of the start of vessel assessment.

    - Conducts the assessment of the vessel before dry docking procedures.

    - Prepares the following documents:

- Survey Form (SF)

- MRS

- PRF

  - Chief Engineer

    - Notes the details indicated in the SF.

    - Updates the Engine Log on repairs and maintenance works to be conducted on vessel.

    - Supervises the conduct of repairs and maintenance on the engine and deck machineries.

    - Prepares the Vessel Maintenance Report (VMR) for every preventive maintenance procedure conducted on engine and deck machineries.

  - Master

    - Notes the details indicated in the SF.

    - Updates the Deck Logbook on repairs and maintenance works to be conducted on vessel.

    - Supervises the conduct of repairs and maintenance on the deck.

    - Prepares the Vessel Maintenance Report (VMR) for every preventive maintenance procedure conducted on the deck department.

    - Summarized the VMR of the engine and deck departments.

    - Prepares the Vessel Repairs and Maintenance Monitoring (VMRR) to be submitted to the Engineering Department on a weekly basis.

  - Hull Supervisor

    - Receives advice from Engineering Manager of Dry Dock Schedule.

    - Supervises Assistant Hull Supervisor in the conduct of vessel assessment upon arrival of the vessel at the dry dock area.

    - Prepares the following documents:

- DDS

- BOM

    - Verifies the following documents:

- SF

- MRS

- PRF

    - Secures the materials, supplies, tools and equipment from the warehouse.

    - Supervises the overall execution of dry dock maintenance.

    - Conducts routine inspection of the quality of the work in progress.

    - Updates the DDS of the ongoing progress of dry dock maintenance.

    - Receives the Progress Report from the Service Provider/Contractor.

    - Reviews and notes the Quality Control Inspection Form (QCIF).

    - Receives the Statement of Account (SOA) from the Service Provider/Contractor.

    - Forwards the SOA to the Accounting and Finance Department for AP Voucher preparation and payment.

  - Electrical Supervisor

    - Verifies the details in the SF regarding electrical equipment and apparatus of vessels.

    - Checks, maintains and repairs electrical equipment of the vessel.

    - Checks, maintains and repairs generator power used in dry dock area.

  - Port Engineer

    - Verifies the details in the SF regarding marine engine, vessel and shore crane, and other on-board machineries.

    - Conducts the routine quality inspection of the ongoing repairs.

  - Quality Control Inspector

    - Perform quality inspection and verifies the details indicated in the Progress Report submitted by the Service Provider/Contractor.

    - Prepares the QCIF and forwards the same to the Hull Supervisor to be duly noted.

- KEY TERMS

  - Afloat Ship Repair -  Repairs onboard a vessel while at sea, at anchorage or at berth which will not require drydocking/shipyard service, subject to vessel safety procedures; Service Contracting refers to similar works provided as contracted by a vessel or a shipyard.

  - Dry Dock – The condition in which a vessel is taken out of water for cleaning and repair of her hull and its integral parts such as rudder, propeller, sea valves and sea chests, among others.

  - Hot Works – Cutting and welding operations that involve the use of portable gas or arc welding equipment, or involve soldering, grinding, or any other similar activities producing a spark, flame, or heat.

  - Rectification – A corrective action on a job conducted on a vessel either due to poor work quality or a deviation from the agreed specifications of the repair works.

  - Sea Trial – The testing phase of a vessel to test and assess the performance of a new vessel or a vessel which had undergone drydocking

  - Shipyard – refers to the shipbuilding or ship repair facilities that have the capability to put ships out of water in a dry-dock or to lift ships above the water surface in order to effect ship work on ships, appendages, structure, machinery and equipment.

  - Survey – Survey shall include a complete inspection of the ship’s structure, machinery and equipment, including the outside of the ship’s bottom and the inside and outside of the boilers to ensure that the arrangements, material and scantling of the structure, boilers and other pressure vessels and their appurtenances, main and auxiliary machinery, electrical installation, radio installations including those used in life-saving appliances, fire protection, fire safety systems and appliances, lifesaving appliances, and arrangements, shipborne navigational equipment, nautical publications, means of embarkation for pilots and other equipment fully comply with the requirements of present regulations, and of the laws, decrees, orders and regulations promulgated as a result thereof by the Administration for ships and the service for which they are intended. The survey is such as to ensure that the workmanship of all parts of the vessel and its equipment is in all respects satisfactory, and that the vessel is provided with the lights, shapes, means of making sound signals and distress signals as required.

  - Tool box meeting – An informal safety meeting that focuses on safety topics related to the specific job, such as workplace hazards and safe work practices. Meetings are normally short in duration and are generally conducted at the job site prior to the commencement of a job or work shift.

  - Vessel – A general term for all craft or artificial contrivance capable of floating in water, designed to be used or capable of being used as a means of water transportation utilizing its own motive power or that of others.

- POLICIES

  - General Policies

    - Only authorized persons are allowed inside the dry dock area.

    - Conduct on repairs. and maintenance shall adhere to the applicable standards of the regulatory agencies (i.e. MARINA, Class Society).

    - As much as possible, the following information should be provided on the Survey Form:

- Maker, type of equipment/machinery, model & serial/engine number

- Details of defects, symptoms, actions already taken to rectify & results

- Any access work, removals, cutting of deck or shell, staging requirements, hot works needed

- Any cleaning works, cooling down, gas-freeing, sludge removal and other procedures conducted before dry dock

    - For the whole duration of the dry dock maintenance, a tool box meeting must be conducted every day before the start of work. The meeting should be properly documented and minutes of the meeting shall be prepared and filed. The following topics should be included in the meeting:

- Progress of the work list items

- Items to be carried out during the day

- Modifications to the work specification

- Review of Safety Management System Manual

    - Any irregularities and problems encountered during dry dock must be immediately reported to the Hull Supervisor.

    - Eating and drinking are only allowed in designated areas inside the dry dock area.

    - Drinking alcohol and smoking are strictly prohibited inside the dry dock area.

    - A new Bill of Materials (BOM) shall be prepared in cases of additional works to be conducted during dry dock maintenance which was not covered by the previous BOM.

  - Preventive Maintenance

    - Vessel inspections shall be conducted on a regular basis. The schedule for the maintenance using the Vessel Maintenance Report (VMR) are as follows: (See Annex B for details)

| Document Used | Interval | Schedule |
| --- | --- | --- |
| VMR No. 1 | Daily | Monday to Saturday |
| VMR No. 2 | Weekly | Every Sunday |
| VMR No. 3 | Monthly | Last three (3) days of every month |
| VMR No. 4 | Quarterly | Every first Monday of: - March - June - September - December |
| VMR No. 5 | Semi-Annually | Every second Monday of: - June - December |
| VMR No. 6 | Annually | Every third Monday of January |
| VMR No. 7 | Running Hours | Depending on the running hours of machineries and equipment |

    - The annual inspection of the vessel shall cover the following:

- Conditions of machinery items: deck and engine, including pipes

- Conditions of vessel structures: deck plating, side and longitudinal frames and vessel side or end ramps/doors;

- Condition of life saving equipment; and

- Further requirements of the vessel in terms of materials and new equipment.

    - The following equipment shall be inspected and tested in accordance with the recommended intervals (see Annex C):

- Emergency steering;

- Emergency lighting;

- Emergency alarm;

- Emergency fire pumps;

- Emergency generator and switchboard; and

- Portable VHF radios.

    - The deficiencies noted, if any, shall be recorded with corresponding corrective actions and brought to the attention of the Operations Manager by means of the Incident Report.

    - The Running Hours Maintenance Schedule shall be checked on a daily basis to schedule target dates to conduct routine maintenance. The Chief Engineer must maintain a record of the running hours of various engine parts.

    - The Chief Engineer shall maintain a record of all pertinent engine-operating parameters on a daily basis. The periodic readings of engine operating parameters for main and generator engines must be taken every 2 hours and must be logged in the appropriate record books. Once a week, the readings taken on a particular voyage must be submitted to the Operations Manager.

    - A semi-annual onboard diagnostic testing must be conducted on the vessel. The diagnostic testing shall be supervised by the vessel’s Chief Engineer. (See Annex D)

    - A lube oil analysis must be conducted on a monthly basis. (See Annex E)

    - On a monthly basis, a water-treatment analysis must be conducted on the vessel. (See Annex F)

    - The Chief Engineer must inform the Master of the start of any major engine maintenance activity so that necessary permits can be secured on time.

    - Major engine maintenance activities must be held only at the port of Cebu, except in unusual circumstances, in order to avail the support of the Company’s machine shop and the services of the skilled machine operator when required.

    - Replaced items that are not repairable are to be returned to the warehouse, accompanied by a duly approved Materials and Supplies Return Slip (MSRS).

    - Replaced items that are repairable are to be brought to the machine shop for repairs. A duly filled-out and approved Work Order Request (WOR) is prepared and forwarded to the machine shop.

  - Dry Docking Works and Repairs

    - Vessels shall be required to dry docked twice within a period of five (5) years. The next scheduled drydocking of a vessel shall be undertaken on the 24th month after the last drydocking. Unless with authorization and approval from MARINA, the scheduled drydocking period (24th month) may be extended to a period not exceeding six (6) months.

    - Three (3) months before scheduled drydocking, the Master and all shipboard department heads must prepare a listing of all work items for the Deck Department based on the Inspection Records and the past entries of the Deck Logbook. The Chief Engineer must similarly do the same for the Engine Department. The Master will review both lists and submits a consolidated list to the Operations Manager. The Work Order (WO) shall be made indicating the list of work items which will be forwarded to the Engineering Department.

    - The Engineering Department will sort the list into two (2) categories – those that can be performed by the Company, and those that can be performed by the Shipyard.

    - The Master will take the vessel to the anchorage premises of the Shipyard to wait for the Yard Dock Master. The vessel is considered to have been delivered to the Shipyard as soon as the Yard Dock Master shall take the vessel to the dock or to any of the Shipyard’s mooring facilities.

    - The Master must see to it that the following details are recorded in the Deck Logbook:

- Departure time

- Arrival time at anchorage

- Arrival of the Yard Dock Master and tugs

- Name of tugs

- Start of maneuvering

- Time secured

    - Vessel assessment shall be conducted three (3) months before scheduled for drydocking as indicated in the WO forwarded by the Operations Department.

    - All required materials and labor must be acquired and contracted, respectively, within three (3) months before dry dock date. Tools and equipment needed for the execution of the dry dock maintenance must be prepared with Property Transfer Slip (PTS) for reservation of the said equipment to be used during dry dock maintenance.

    - Details of the drydocking must be regularly communicated to the Operations Department to facilitate updating of details in the Vessel Monitoring System.

    - For instances of being aground or during collisions, an Incident Report and Letter of Protest shall be immediately forwarded to the Operations Department. vessel assessment shall be conducted immediately upon availability of responsible personnel and required permits.

    - After the conduct of the dry dock maintenance, a sea trial shall be conducted in which as Sea Trial Report is made. The sea trial shall be conducted only by authorized personnel/crew to board the vessel. Moreover, a tank sounding shall be conducted before and after the sea trial.

  - Afloat Vessel Repairs

    - The Engine Officer on Watch (EOW) will stop the engine once the defect has manifested itself and is likely to endanger the continued operation of the engine.

    - The EOW will initiate corrective action based on the prevailing engine conditions at the time of the breakdown.

    - The EOW will inform Officer on Watch of the engine breakdown

    - All engine personnel must assist and give due support to the Chief Engineer during breakdown at sea.

    - Corrective actions taken shall be duly indicated in the Running Hours Maintenance Schedule.

  - Repairs and Maintenance Work Inspection

    - The Master will conduct daily work inspections to ensure that work executed by the Shipyard is in accordance with the work specifications submitted.

    - Daily progress of the repairs and maintenance work shall be reflected in the Deck Maintenance Record Books and the Engine Log Book by the Master and Chief Engineer, respectively.

    - The following shall be considered by the Master on ongoing painting procedures:

- Ensure that surfaces to be painted are to be cleaned dry and in-line with the surfaces specification including the internal surfaces of the sea chest boxes;

- Check the prevailing weather conditions before and during every painting activity is done:

  - NO painting shall be started when it is raining or when the relative humidity is 85% or as specified by the paint manufacturer;

  - NO painting shall be started when the temperature of the hull to be painted is less than 2 degrees Celsius above the dew point or as prescribed by the manufacturer; and

  - NO painting shall continue at the onset or when the relative humidity has reached 85% or as specified by the paint manufacturer.

    - The following shall be considered by the Master on ongoing steel repairs:

- The Master shall make sure that specified frames, stiffeners, brackets, floors, girders, deck plates, side plating, hull painting, or portions thereof are exactly removed or cut by the Shipyard in accordance to the work specification submitted;

- Check that new replaced plates or shapes are of the correct specification; and

- Check that all joints are welded free of dirt, cracks or undercuts.

    - Water-tightness of internal compartments of the vessel will be ascertained upon completion of structural repairs using hydro testing or air testing.

    - Inspections on the validity of the Progress Billing are to be carried out by the Quality Control Inspector, together with the Engineering Department’s Supervisors (i.e. Hull Supervisor, Port Engineer and Electrical Supervisor).

    - Adequate function tests and inspections are to be carried out for all safety systems that had been disturbed during the repair period.

    - All critical equipment, such as key navigational, propulsion, steering, cargo, ballast, fire and gas detection systems are to be tested before completion.

    - In cases of discrepancy between the Progress Report and the Quality Inspection conducted by the Quality Control Inspector, a Rectification Notice (RN) is prepared. This shall be addressed to the Service Provider/Contractor for acknowledgement of rectification to be done.

    - Repairs and maintenance works which do not pass the inspection shall be communicated to the Service Provider/Contractor. Discrepancies in work done shall be immediately rectified before processing of Statement of Account.

  - Safety Precautions

    - Before execution of dry dock maintenance, personnel must wear proper personal protective equipment (i.e. hard hat, safety shoes, safety gloves, safety goggles, face mask, welding shield, etc.).

    - All navigational equipment (i.e. radar, GPS, gyrocompass, etc.) must be turned off during hot works.

    - Battery unit must be disconnected during execution of welding works.

    - Flammable materials must be kept distant from ongoing hot works.

    - A fire watchman must be present throughout the whole duration of the dry dock maintenance.

    - Sufficient number of suitable emergency routes and exits that must be kept clear and free from obstruction and, where necessary, provided with emergency lighting so that it may be used at any time.

    - Where necessary suitable and sufficient fire-fighting equipment and fire detection and alarm systems must be installed.

    - Workers must be instructed in the correct use of fire-fighting equipment.

    - Appropriate precautions must be taken when using hot work in the vicinity of flammable materials. Where possible any combustible items should be removed before the hot works commence and if this is not practicable then combustible items should be wetted.

    - A proper advice must be communicated to all personnel within the affected area of any planned usage of lifting equipment.

    - The Dry Dock Area shall always be kept clean and tidy.

    - Equipment not in use should be stored properly in the warehouse.

    - Final cleaning of machinery spaces, accommodation and other areas where repairs have been carried out is to be completed as soon as possible after departure.

  - Fuel Monitoring

    - Tank sounding shall be conducted on a regular basis and attended by two representatives from the following:

| Deck Department | Master / Captain |
| --- | --- |
|  | Chief Mate |
| Engine Department | Chief Engineer |
|  | Oiler |

The fuel monitoring in-charge/maintenance staff may assign a vessel personnel to witness the sounding procedures and report directly to the fuel monitoring in-charge his observations on the completeness, reliability and accuracy of the sounding procedures.

At a minimum, tank sounding shall be conducted daily at port or anchorage, prior to departure at the port of origin and upon arrival at the port of destination. Tank sounding is also conducted before and after refueling. Moreover, unannounced sounding inspection shall be conducted.

    - The tank sounding shall be recorded in the Tank Sounding Record (TSR) which shall then be forwarded to the Maintenance Staff for monitoring and evaluation.

    - The Maintenance Staff shall prepare and maintain a master list of all vessels with the following details:

- Number of fuel tanks, including lube and other oils

- Location of each tanks

- Maximum capacities of each tank

- Minimum fuel level of each vessel (replenishment point)

- Power stroke (ratings) of each tank

- Revolution per minute (RPM) of each tank

- Average speed of each tank (knots)

- Average fuel consumption per hour per main engine and auxiliary engine on each vessel

- Average fuel consumption per voyage based on distance in nautical miles

- Average fuel consumption per port based on daily activities

- Variance analysis (actual consumption versus pre-determined/standard consumption)

    - A Vessel Fuel Consumption Report (VFCR) shall be prepared by the Maintenance Staff after every voyage to account for the total consumption of each vessel at the following points:

- Port consumption at the port of origin (during loading activities)

- Voyage consumption (steaming)

- Port consumption at the port of destination (during unloading activities)

    - At each month end, the Maintenance Staff shall prepare a report to the Accounting Department detailing the fuel movements of each vessels during the month, as follows:

- Beginning fuel inventory onboard

- Total fuel purchases during the month

- Total fuel consumption during the month

- Ending fuel inventory onboard

    - The bunkering protocol shall require presence of the following Officials and Crew while vessel is refueling, to wit:

- Captain/Master

- Chief Mate

- Deck Apprentice

- Chief Engineer

- Second Engineer

- Lower Deck Apprentice

The Chief Engineer shall entirely be responsible for this activity requiring those above mentioned to witness loading and CE’s signing the delivery/receiving report.

    - Chief Engineer must not receive the delivery if he is in doubt of the tank’s seal integrity, or if there is a discrepancy in delivery (may it be in quantity or quality issue) of fuel, and must report immediately to Port Captain/ Operations/ Makati Purchasing.

Furthermore, witnesses shall closely inspect delivery truck for possible fuel retention and Chief Engineer should require truck to be tilted, one side after another, to ascertain that there is no fuel retention in the presence of above witnesses.

    - Where bunkering is outside of Cebu/Manila and submission of the delivery/ receiving report is not possible, Chief Engineer shall send a picture or scanned copy of the report to Purchasing Department-Makati, copy furnished Engineering & Maintenance. Thereafter, documents should be mailed through fastest means to Purchasing Department-Makati.

  - Documentation

    - Business forms (i.e. Survey Forms, Bill of Materials, Quality Control Inspection Form) shall be prepared/issued sequentially.

    - All records shall be neatly and legibly kept in ink to prevent alteration of records.

    - Business forms shall be duly filled-out and signed by designated signatories. Any details/signatories which are not applicable must be duly indicated with “N/A”.

    - If designated signatory is unavailable, printed evidence of consent (i.e. text, emails, etc.) shall be attached to the business form which should be post-approved by the designated signatory.

    - Alterations, if any, shall be duly countersigned.

  - Handling of Tools and Equipment

    - Only authorized maintenance staff shall issue the materials, tools and equipment.

    - While the dry dock maintenance is in-progress, any unused materials should be properly arranged/neatly stacked in the designated areas in the Shipyard and must be labeled to ease identification.

    - All tools and equipment shall be handled with care. Any plugged equipment should be unplugged after its use.

    - For new equipment, a run-through (pilot test) shall be made in the presence of the Hull Supervisor and all other engineering personnel expected to use the equipment. The manual of the equipment shall be considered.

    - No tools or equipment shall be taken out of the dry dock area without the permission of the Hull Supervisor.

    - Any sign of pilferage, theft, robbery, loss or untoward incidents shall be reported immediately to the Hull Supervisor.

    - All unused materials, and tools and machineries used in the conduct of the repair shall be returned immediately to the Warehouse/Tool room. The return of the unused materials and supplies shall be returned with duly approved Materials and Supplies Return Slip (MSRS). The tools and machineries shall be returned to the Tool Room together with the previously received copy of the Property Transfer Slip (PTS).

  - Ship Crane Handling

    - The Chief Engineer has the overall responsibility for ensuring that the vessel's cargo cranes are being adequately maintained and that he receives regular updates from the Second Engineer and Crane Operator on their condition.

    - Any defects which affect the vessel’s operation or efficiency are to be reported immediately to the Operations Department, in coordination with the Engineering Department, who will decide what course of action is to be taken.

    - The Chief Engineer shall be responsible for all crane maintenance. Under his supervision, the Second Engineers and Crane Operator must carry out all necessary maintenance to ensure that the vessel’s crane are in good working condition during the loading and unloading operations.

    - It is the joint responsibility of the Chief Engineer and the Chief Mate to ensure that all moving parts are adequately greased, machinery being the responsibility of the Second Engineer and the wires, sheaves, etc. being the responsibility of the Crane Operator.

There must be a close communication between the officers and Crane Operator to determine a proper and suitable grease program, taking into consideration the amount of use the cranes have been or will be subjected to. General maintenance of the crane exteriors, mechanism and fittings is the responsibility of the Crane Operator.

    - Repair to crane jibs must be carried out only in consultation with the Operations/Marketing and Maintenance Department. Any damage noticed to crane jibs must be reported immediately to the Port Engineer, Maintenance Department, ship owner/manager and advice sought before carrying out any kind of repair. All repairs and works to crane must be properly coordinated to avoid unnecessary delay or disruption to operations.

    - Good quality wire grease must be regularly applied to provide lubrication and to protect the crane from weather conditions affecting the safe operations of the cranes. Wire ropes must be replaced if there are breaks on strand, kinks and excessive wear.

    - During wire greasing procedures, attention must be paid to certain parts of the wires, which are less accessible (i.e. hoist and luff wires which lie on the sheave, and those which lie inside the crane structure), where they are secured. All pins and pulley sheaves with grease fittings must be regularly applied with grease to minimize wear on bearings and bushings.

    - Over-greasing of crane wires must be avoided especially during very warm weather conditions. If the crane wires are over-greased, it is possible that the grease may begin to run and drop onto the cargo. This must also be avoided as it may result in cargo damage claims.

    - Crane jibs/boom must be lowered into the boom rest and secured properly before voyage. The electric plugs and sockets are to be covered against entry of water, and the electric cable is to be hove sufficiently tight to prevent chaffing when the vessel is working in a seaway.

    - All crane windows, doors and ventilation hatches are to be closed and any portable guardrails and/or chains to be replaced. If the vessel carries grabs or other cargo handling equipment, they shall be well secured on every occasion before the vessel proceeds to sea. Under no circumstances should they be left unsecured, even during short coastwise sea passage.

    - The safe working load (SWL) must be clearly marked in a conspicuous place in the crane jib/boom, and it must be ascertained by the Chief Officer that Crane Operator and to all parties concerned with the load and unloading operation are aware of the maximum capacity of cranes, and that this is not exceeded. The weight of grabs, spotters, or other cargo handling equipment attached to the hook must always be taken into account.

    - During repairs, ensure that all controllers are placed in the “off” position, and the main switches are open and locked. Place on the switch, a standard warning tag stating “DO NOT START”.

    - Carrying of items using the hands when going up and down ladders during repairs is prohibited. Items that are too large to go into pockets or belts should be lifted to or lowered from the crane by a rope.

    - The area below the crane being repaired must be cleared and a barrier erected to prevent injury from a falling object.

    - Replace all guards and other safety devices before leaving a crane. Moreover, remove all stops, tools, loose parts and other material and dispose of them before completing the repair job.

- PROCEDURES

| Vessel Assessment |  |  |  |
| --- | --- | --- | --- |
| Step No. | Activity | Personnel Involved | Business Forms |
| 1 | Receives duly accomplished and approved WO from Operations Department. | Engineering Manager | Approved WO |
| 2 | Coordinates with Hull Supervisor of schedule for dry dock maintenance. | Engineering Manager |  |
| 3 | Designates Assistant Hull Supervisor for the execution of vessel assessment. | Hull Supervisor |  |
| 4 | Conducts assessment of vessel and duly fills-out the Survey Form (SF). The Assistant Hull Supervisor signs the “Prepared by” portion of the SF. | Assistant Hull Supervisor | Duly filled-out SF |
| 5 | Forwards duly filled-out SF to Chief Engineer to be duly noted. | Assistant Hull Supervisor |  |
| 6 | Duly notes SF and updates Engine Log of details of assessment and work to be done. Forwards the same to the Master. | Chief Engineer | Engine Log |
| 7 | Duly notes SF and updates Deck Logbook of details of assessment and work to be done. Forwards the same to the Hull Supervisor, Port Engineer and Electrical Supervisor for verification. | Master | Deck Logbook |
| 8 | Verifies the details indicated in the SF and returns the same to the Assistant Hull Supervisor for the preparation of the BOM. | Hull Supervisor, Port Engineer and Electrical Supervisor | Verified SF |

| Schedule and Budgeting |  |  |  |
| --- | --- | --- | --- |
| Step No. | Activity | Personnel Involved | Business Forms |
| 1 | Obtains duly verified SF, and prepares Dry Dock Schedule (DDS). | Hull Supervisor | DDS |
| 2 | Forwards DDS to the Engineering Manager for verification. | Hull Supervisor |  |
| 3 | Verifies the DDS. | Engineering Manager | Verified DDS |
| 4 | Prepares the Bill of Materials (BOM) and forwards the same to the Engineering Manager for approval. | Hull Supervisor | BOM |
| 5 | Verifies the details in the BOM and returns the same to the Hull Supervisor. | Engineering Manager | Verified BOM |
| 6 | Receives duly verified BOM and forwards document to Warehouse Department to obtain data of availability of items required in the BOM. | Hull Supervisor | Verified BOM |
| 7 | Obtains copy of BOM and check on the availability of requested items in the Warehouse System. Inputs the data of the available quantity and current unit cost in the BOM. Warehouse Supervisor signs the “Data provided by” portion of the BOM. | Warehouse Department | Duly filled-out and verified BOM |
| 8 | Obtains copy of BOM and perform canvassing of unavailable items for procurement. Inputs the data of the current unit cost of the items to be purchased in the BOM. Purchasing Manager signs the “Data provided by” portion of the BOM. | Purchasing Department | Duly filled-out and verified BOM |
| 9 | Receives duly filled out copy of BOM and forwards the same, together with duly verified DDS and SF, for approval. | Hull Supervisor | Duly filled out BOM, DDS, and SF |

| Requisition Procedures |  |  |  |
| --- | --- | --- | --- |
| Step No. | Activity | Personnel Involved | Business Forms |
| 1 | Obtains copy of the BOM, DDS, and SF. | Assistant Hull Supervisor | BOM, DDS, SF |
| 2 | Prepares requisition forms to be forwarded to respective departments and forwards to the Hull Supervisor for verification. For requisition of materials and supplies available at the Warehouse, prepares Materials Requisition Slip (MRS) in two (2) copies. For procurement of materials and services, prepare Materials/Services Procurement Requisition Form (MSPRF) in two (2) copies. | Assistant Hull Supervisor | MRS; MSPRF |
| 3 | Verifies the MSMRS and PRF and forwards the documents to the Engineering Manager for approval. | Hull Supervisor | Verified MRS and PRF |
| 4 | Receives and approves MRS and MSPRF. Returns the same to the Assistant Hull Supervisor. | Engineering Manager | Approved MRS and MSPRF |
| 5 | Obtains copies of duly approved MRS and MSPRF. | Assistant Hull Supervisor | Approved MRS and MSPRF |
| 6 | Forwards the MRS to the Warehouse Department for issuance of materials and supplies. See process title “Warehousing and Issuance of Materials and Supplies” for requisition process. Forwards the MSPRF to the Purchasing Department for procurement of materials, supplies and services. See program title “Procurement of Materials and Services” for procurement process. | Assistant Hull Supervisor |  |

| Execution and Inspection of Repairs and Maintenance |  |  |  |
| --- | --- | --- | --- |
| Step No. | Activity | Personnel Involved | Business Forms |
| 1 | Receives notice from Operations Department of start of execution of dry dock maintenance and assigns Assistant Hull Supervisor to secure ordered materials and supplies from Warehouse. | Hull Supervisor |  |
| 2 | Secures materials, tools and equipment from the Warehouse/Tool Room. | Assistant Hull Supervisor |  |
| 3 | Signs the Materials and Supplies Issuance Slip (MSIS) and the Accountability Form (AF) from the Warehouse/Tool Room. | Assistant Hull Supervisor | Duly received MSIS and AF |
| 4 | Perform execution of dry dock maintenance. | Maintenance Department |  |
| 5 | Conducts routine inspection of ongoing dry dock maintenance on vessel and for additional work, materials, tools and equipment needed. For additional work to be conducted on the vessel, a new BOM is prepared. New requisition forms are prepared for additional materials, services and tools needed. Updates the Dry Dock Schedule of the current progress of the dry dock maintenance. | Assistant Hull Supervisor |  |
| 6 | Receives progress report from Service Provider/Contractor. | Hull Supervisor |  |
| 7 | Conducts physical inspection and verification of details in the Progress Report. | Quality Control Inspector |  |
| 8 | Fills-out QCIF. Signs the “Prepared by” portion of the QCIF. | Quality Control Inspector | Duly filled-out QCIF |
| 9 | Forwards duly filled-out QCIF to the Hull Supervisor for notification. | Quality Control Inspector |  |
| 10 | Signs the “Noted by” portion of the QCIF after verification of details and forwards the same to the Contractor for acknowledgement. | Hull Supervisor | Duly noted QCIF |
| 11 | Acknowledges the details indicated in the QCIF. If the result of the inspection revealed works requiring rectification, a Rectification Notice is prepared to be acknowledged by the Service Provider/Contractor. If Percentage of Completion in Progress Report is 100%, the quality control inspection shall be equivalent to a final function test and inspection of the vessel. Moreover, the QCIF and Progress Report shall be temporarily filed, awaiting the Work Done (WD) / Work Accomplishment Report (WAR) from the Service Provider/Contractor. | Service Provider/ Contractor | Duly acknowledged QCIF |
| 12 | Receives the Statement of Account and forwards the same, together with the Progress Report and the duly acknowledged QCIF, to the Accounting Department for A/P Voucher preparation and payment. | Hull Supervisor | BOM |
| 13 | Perform final function tests and inspection of vessel. | Hull Supervisor |  |
| 14 | Prepare Certificate of Completion (COC) and provide copy to Service Provider/Contractor. Retains one copy of the COC. | Hull Supervisor | COC |
| 15 | Receive copy of WD / WAR from Service Provider/Contractor. Forwards WD/WAR together with SOA, COC, QCIF, Progress Report to the Accounting Department for A/P Voucher preparation and payment. | Hull Supervisor |  |

- FLOWCHARTS

  - Vessel Assessment

  - Schedule and Budgeting

  - Requisition Procedures

  - Execution of Repairs and Maintenance

  - Quality Control Inspection

- BUSINESS FORMS

  - Survey Form

Page 01 of 06

Page 02 of 06

Page 03 of 06

Page 04 of 06

Page 05 of 06

Page 06 of 06

No. of Copies/color	-	1 (white)

Explanation			-	this is prepared every time a vessel assessment is conducted before dry docking maintenance

Prepared by	-	Assistant Hull Supervisor

Noted by	-	Master

Chief Engineer

Verified by	-	Hull Supervisor

Port Engineer

Electrical Supervisor

  - Dry Dock Schedule

No. of Copies/color	-	1 (white)

Explanation			-	this is prepared to document the target schedule of dry dock maintenance to be conducted on vessel

Prepared by	-	Hull Supervisor

Verified by	-	Engineering Manager

Approved by			-	President

Distribution                	-	Engineering Department (attached to the BOM, together with the SF)

  - Bill of Materials

No. of Copies/color	-	1 (white)

Explanation			-	this is prepared to indicate the materials, service and equipment needed for the dry dock maintenance

Prepared by	-	Hull Supervisor

Verified by	-	Engineering Manager

Data Provided by	-	Warehouse Department (warehouse items)

Purchasing Department (items for procurement)

Approved by	-	President

Distribution                	-	Engineering Department (filed chronologically together with the DDS and SF)

  - Bill of Materials Summary

No. of Copies/color	-	1 (white)

Explanation			-	this is prepared to document the work to be done and to summarize the details of expenses indicated in the BOM. This is presented to the President for approval of the BOM and the DDS

Prepared by	-	Engineering Manager

Noted by	-	President

Distribution                	-	President

  - Quality Control Inspection Form

No. of Copies/color	-	2

Explanation			-	this is prepared upon verification of the details indicated in the Progress Report

Prepared by	-	Quality Control Inspector

Verified by	-	Hull Supervisor

Checked by	-	Foreman

Acknowledged by		-	Service Provider/Contractor

Distribution                	-	QCIF 1	-	Accounting Staff (attached to the Check Voucher together with the Progress Report, Statement of Account and RN, if any)

QCIF 2	-	Service Provider/Contractor

  - Rectification Notice

No. of Copies/color	-	2

Explanation			-	this is to document details of rectification to be conducted on vessel after inspection

Inspection conducted by	-	Quality Control Inspector

Verified by	-	Hull Supervisor

Approved by	-	Engineering Manager

Acknowledged by		-	Service Provider/Contractor

Distribution                	-	RN 1	-	Accounting Staff (attached to the Check Voucher together with the QCIF, Progress Report and Statement of Account)

RN 2	-	Service Provider/Contractor

  - Sea Trial Report

No. of Copies/color	-	3

Explanation			-	this report is prepared to document the execution of sea trial after the execution of dry dock maintenance to verify that the vessel is running in good condition

Distribution                		-	STR 1	-	Shipyard’s (service provider) copy

STR 2	-	Vessel’s copy

STR 3	-	Engineering Department (filed per vessel)

  - Certificate of Completion

No. of Copies/color	-	2

Explanation			-	this is prepared to certify the 100% completion of the work done by the Service Provider/Contractor

Distribution                	-	COC 1	-	Accounting Staff (attached to the Check Voucher together with the QCIF, Progress Report and Statement of Account)

COC 2	-	Service Provider/Contractor

  - Tank Sounding Record

No. of Copies/color	-	1

Explanation			-	this is prepared to record the tank soundings conducted on the vessel

Distribution                	-	Vessel’s file copy

  - Vessel Fuel Consumption Report

No. of Copies/color	-	3

Explanation			-	this is to monitor the fuel consumption of every vessel per voyage

Prepared by	-	Maintenance Staff

Verified by	-	Port Engineer

Noted by	-	Engineering Manager

Distribution                	-	VFCR 1	-	Engineer Department

VCFR 2	-	Accounting Department

VCFR 3	-	Operations Department

  - Work Order

No. of Copies/color	-	1

Explanation			-	this is prepared to request engineering and maintenance assistance on vessel

Prepared by			-	Designated Operations Personnel

Approved by			-	Operations Manager

Distribution                		-	Engineering Department (attached to COC)

  - Vessel Maintenance Report (VMR)

Daily VMR

Weekly VMR

Monthly VMR (page 1 of 2)

Monthly VMR (page 2 of 2)

Quarterly VMR

Semi-Annual VMR

Annual VMR

VMR by Running Hours

No. of Copies/color	-	2

Explanation			-	this is to document details of inspection and maintenance on vessel deck and engine conducted by the designated vessel and engine crew on set intervals

Inspection by	-	Designated vessel crew

Noted by	-	Master and Chief Engineer

Distribution                	-	VMR 1	-	Engineering Department (filed per vessel, to be attached to the VRMM)

VMR 2	-	Vessel (file copy)

  - Maintenance Inspection Sheet

No. of Copies/color	-	1

Explanation			-	This is used to document the inspection conducted on the compliance of the vessel crew with maintenance schedule provided.

Inspected by	-	Quality Control Inspector

Conformed by 	-	Master

Chief Engineer

Distribution                	-	Engineering Department

  - Vessel Repairs and Maintenance Monitoring

No. of Copies/color	-	1

Explanation			-	this is used to summarize all maintenance works conducted on the vessel and report any significant repair works done after inspection. This form is to be submitted by the Master to the Engineering Department on a week basis.

Distribution                	-	Engineering Department

  - Running Hours Maintenance Schedule

No. of Copies/color	-	1

Explanation			-	this is used as monitoring of the corrective and preventive maintenance conducted on vessel.

Distribution                	-	Vessel copy

- EFFECTIVITY

This Policies and Procedures Manual shall take effect upon approval and shall supersede any memorandum/SOP inconsistent with this Policies and Procedures Manual. Any changes to the manual shall comply with the policies and procedures indicated in the process title “Amendment of Manual”.

ANNEX

ANNEX A

TANK SOUNDING GUIDELINES

When to Take Manual Sounding?

Manual sounding is taken by the vessel’s crew on a daily basis, but the following are the operations and situation wherein the crew has to take manual sounding without fail:

    - On a daily basis to check the level of all vessel’s tanks

    - When Oil Transfer Operation is going on, for e.g. bunkering, sludge or bilge transfer, etc.

    - When fresh water is received from port

    - When Oily Water Separator (OWS) is operated, bilge tank sounding is monitored manually

    - When vessel is involved in grounding or collision, the officers from each department must check all the tank sounding to assess for any leakages

    - When Chief Engineer is relieved by the new Chief, he/she should check the actual level and oil quantity present onboard by manually sounding the tanks

    - Before the vessel is reaching a port, all important oil tanks are sounded and quantity cross checked with sounding records for making the cargo plan

    - When vessel is going to a dry dock for drydocking

    - Ballast tanks sounding before approaching port, canal, channel, etc.

What is the Frequency in taking Manual Sounding?

Sounding frequency on a vessel differs from company to company and according to the working policy and the nature of operations going on board. A brief note as per the general procedure followed on vessel is given below:

- All Fuel oil tanks lube oil tanks and diesel oil tanks must be sounded twice a day, once in morning and once in evening, and recorded in the event of leak or any other emergency related to oil content of tanks.

- All waste water tanks including bilge and sludge tank must be sounded twice a day.

- Ballast water tank and fresh water tanks should be sounded once a day.

- As bunkering operation is one of the most critical operations, the frequency of operation drastically increases at the initial and final stages of the operation (every 15 minutes) to check the inflow of oil in correct tank at initial stage and to avoid overflow of oil during final stage of operation.

- When waste water of vessel is internally transferred, discharged overboard through 15 ppm equipment or given to the shore facility, sounding is to be taken twice or thrice every hour.

- When ballasting or de-ballasting of the tank is going on, hourly sounding is to be taken or as decided by the chief officer.

- Fresh water tank sounding to be taken daily. Feed water tanks in the engine room are to be sounded at every watch to keep a track of leakages.

What are the Tools Used for Sounding?

The following basic tools and things are used for manual sounding:

- Sounding Tape

For manual measurement of sounding, a measuring tape normally made up of brass and steel with a weighted bob attached at the end of the tape using a strap hook. Sounding Tape comes in meter or feet metric with bob is mostly of cylindrical shape and Torpedo shape. Torpedo (Dip-weight in the form of a rod, terminating in a truncated cone at the lower end) shape is more commonly used for thicker liquid as the cone penetrates the oil/sludge more easily.

A stainless steel tip on the brass dip-weight prevents wear down. Cylindrical bob is used for sounding of water and diesel/ lube oil and similar fluids where penetration is easy to achieve.

Stainless steel dip-weights are intended to use with stainless steel tapes. Brass dip weights are to be used with carbon steel and polyamide coated tapes.

Generally, the dip weights available for measuring water and similar fluids is approx. 400 grams. Bob weight of 700 grams to 1000 grams are most common one for oils and fats.

- Water / Oil Finding Paste

These are special pastes which are applied on the sounding tape. They are normally available for water sounding and gasoline/diesel oil sounding as they are transparent in color and difficult to detect in the measuring tape. The paste is applied on the sounding tape and when it comes in contact with the water or gas oil, it will change its color, indicating the level.

- Sounding Table

Sounding table is a chart describing the construction, capacity and most importantly the volumetric content of the tank at given depth of sounding or ullage. All vessels have their own sounding table documents for each tank containing fluid in bulk. The sounding table is compiled to show the volumetric quantity of fluid at various trims and list for the particular sounding depth in cm.

How to take Manual Sounding?

There are two methods to measure level of the tank:

- Measuring Level by Sounding the Tank

In this method, the sounding tape is inserted in the sounding pipe and bob is lowered till it touches the bottom of the tank i.e. the datum plate (check figure). The tape will measure the complete depth of the liquid which will be compared to the sounding value in the sounding table.

- Measuring the Level by Calculating the Ullage of the Tank

When measuring level by ullage method, the sounding tape is inserted only till the bob/ tape touches some part of the fluid top surface. Instead of measuring the whole depth of the tank, only the free space (from the top of sounding pipe to liquid surface) is measured (Check Figure). This method is convenient if the fluid in the tank is at a higher level. In heavy oil tanks and viscous fluids, this method does not spoil the whole sounding tape and avoids a false reading.

For e.g. If the sounding tape is lowered till 500 cm and the tape measures oil in the tape as 100 cm. The total depth of the sounding pipe is 1600 cm. Hence the ullage is measured as

1600 – 500 +100 = 1200 cm.

Normally ullage values are also provided in the sounding table with reference to the trim of the vessel to get the volumetric content in the tank.

Follow the steps mentioned below to take the sounding on a vessel using the sounding tape:

- Make sure the bob is tightly held with the tape using a strap hook. Ensure that the tape is not damaged anywhere in between to avoid dropping of bob or tape inside the pipe.

- Know the last reading of the tank in order to have a rough idea whether to take sounding or ullage.

- Apply water/oil finding paste to get exact readings (for water and gas oil)

- Drop the tape inside the pipe and make sure it strikes the striker plate.

- Coil up the tape and check for impression of paste and then note the sounding.

- Check the trim and list of the vessel to read the correct reading of volumetric content of the vessel.

- Note down the sounding in the record books with signature of the officer in charge.

ANNEX B

FREQUENCY OF INSPECTION / MAINTENANCE

  - Life Saving Equipment

| Items | Frequency |
| --- | --- |
| Lifejackets | Monthly |
| Lifebuoy – Unlighted | Semi-annual |
| Lifebuoy – Lighted | Weekly |
| Lifebuoy – Lighted with smoke signal (MOB) | Weekly |
| Life rafts – inflatable | Weekly |
| First aid chest (for freighter vessels) | Weekly |
| Pyrotechnic – Hand Flares | Every Abandon Ship Drill* |
| Pyrotechnic – Rocket Parachute Signal | Every Abandon Ship Drill |
| Pyrotechnic – Smoke Signal | Every Abandon Ship Drill |
| Dispensary and mechanical supplies | Monthly |
| Two-way VHF Radiotelephones |  |

  - Fire Fighting Equipment

| Items | Frequency |
| --- | --- |
| Fire Ax | Every Fire Drill* |
| Fire Hatchet | Every Fire Drill |
| Fire Buckets | Every Fire Drill |
| Fire Hose, Nozzle and Box | Every Fire Drill |
| Fireman’s Outfit | Every Fire Drill |
| Fire Alarm System – Smoke Detector | Every Fire Drill |
| Fire Alarm System – Fire Detector | Every Fire Drill |
| Fire Alarm System – Fire Alarm Klaxon | Every Fire Drill |
| Fire Extinguisher (portable) | Monthly |
| CO2 Smothering System | Annually |
| Fire Pumps – Main | Monthly |
| Fire Pumps – Emergency | Monthly |
| Ventilation Cowl | Monthly |
| Heads and Closing Arrangement | Monthly |
| Watertight Doors of Bulkheads | Monthly |
| Emergency Stop Control | Quarterly |
| Quick Closing Valves | Quarterly |
| Escape Trucks | Monthly |
| Fire Doors | Every Fire Drill |
| Communication System | Monthly |
| Ventilation Motors | Monthly |

* Abandon Ship Drill and Fire Drill shall be conducted on a monthly basis.

  - Navigational Equipment

| Items | Frequency |
| --- | --- |
| Radars | Daily |
| Master Gyro and Repeaters | Daily |
| Standard Magnetic Compass | Daily |
| Steering Magnetic Compass | Daily |
| Rudder Indicator | Daily |
| Speed Log (if any) | Daily |
| Whistle | Daily |
| Course Recorded (if any) | Daily |
| Chronometer | Daily |
| Radio Direction Finder (DRF) (if any) | Daily |
| Binoculars | Daily |
| Signaling Lamp | Daily |
| Navigational Lights | Semi-annually |
| Shapes and Gongs | Daily |
| Steering Mechanism | Monthly |
| Clear View Screen | Daily |
| Barometer | Daily |
| Thermometer | Daily |
| Wind Vane | Daily |
| Anemometer | Daily |
| Pelorus Stand | Daily |
| Azimuth Circle | Daily |

  - Communication Equipment

| Items | Frequency |
| --- | --- |
| Main Transmitter and Receiver | Daily |
| Emergency Batteries | Weekly |
| Handheld Radios | Every Abandon Ship |
| VHF Radio | Daily |

  - Structural Elements

| Items | Frequency |
| --- | --- |
| External Hull, wherever possible | Monthly |
| Superstructure plating, including windows and other openings | Monthly |
| Topside Plating | Monthly |
| Weather Deck Plating and Drainage (for freighter) | Before and After Loading |
| Hatchways | Monthly |
| Bulkheads, Frames, Stiffeners and Pillars within Cargo Spaces | Monthly |
| Tank Top Plating | Semi-annually |
| Air Vent Pipe and Sounding Pipe | Semi-annually |
| Ventilation Trunks | Every Fire Drill |
| Manhole Covers | Monthly |
| Ballast Tanks and Pipe | Semi-annually |
| Fresh Tanks and Pipe | Annually |
| Cofferdams | Annually |
| Bulwarks, Railings, Fairlead Rollers and Bollards | Monthly |
| Locker Compartments | Monthly |
| Mast and Mast Houses | Monthly |
| Derricks and Accompanying Runners (if any) | Monthly |

  - Deck Machinery

| Items | Frequency |
| --- | --- |
| Operational Test | Daily before maneuvering |
| Checking for Oil Leakage | Daily |
| Oil Level Check, fill reservoir if needed | Weekly |
| Checking of Couplings of Pumps and Drive Motors | Weekly |
| Lubrication at Grease Points | Monthly |
| Surface Preparation and Preservation for Surfaces Exposed to Weather | Monthly |
| Checking and Lubricating Wire Ropes | Monthly |
| Checking and Greasing Blocks and Pulleys | Monthly |
| Replacement of Wire Ropes | Every 4 to 6 months |

  - Ancillary Equipment

Freshwater and Seawater Pumps

| Items | Frequency |
| --- | --- |
| Operational Test | Daily |
| Greasing of Bering Housing (if applicable) | Weekly |
| Checking Coupling of Pump and Drive Motor | Monthly |
| Packing (or Seal) of Gland Housing | As Needed |
| Re-tightening of Packing Presser (if applicable) | As Needed |
| Inspection of Internal Parts: - Impeller - Bearings - Shaft - Bearing Rings - Internal of Housing | Quarterly |
| General Overhaul | Annual |
| Checking Bearings of Electrical Motor | Semi-Annually |

Fuel Oil, Lube and Hydraulic Pumps

| Items | Frequency |
| --- | --- |
| Operational Test | Daily Before Maneuvering |
| Checking of Shaft Seal for Leakage | Daily Before Maneuvering |
| Checking Coupling of Motor and Pump | Monthly |
| Checking Bearings of Electrical Motor | Semi-Annually |

Purifiers

| Items | Frequency |
| --- | --- |
| Operational Test | Daily Before Maneuvering |
| Checking and Cleaning Water and Oil Impellers | Every 2,000 hours |
| Checking Disk and Checking for Corrosion | Every 2,000 hours |
| Checking and Cleaning Bowl Body, Hood and Distributor | Every 2,000 hours |
| Checking and Cleaning Vertical Shaft with Spiral Gear | Every 2,000 hours |
| Checking Friction Clutch | Every 8,000 hours |
| Checking of Water Chamber, Operating Water Disc and Distributing Nozzle | Every 6,000 hours |
| Checking Oil Level | Daily |
| Replacing Bearings (Internal) | Every 8,000 hours |
| Replacing Lube Oil | Every 2,000 hours |
| Checking of Bearings of Electrical Motor | Semi-Annually |

Air Compressor (Main and Emergency)

| Items | Frequency |
| --- | --- |
| Operational Test | Daily |
| Checking Oil Quantity | Daily |
| Checking of Pistons, Rings, Pin and Brush | Every 1,000 hours |
| Renewing of Piston Rings | Every 1,000 hours |
| Checking of Connecting Rod and Bearing | Every 1,000 hours |
| Checking of Crankshaft and Main Bearing | Every 2,000 hours |
| Checking of Mechanical Seal | Every 1,000 hours |
| Checking of Pipes for Leakage | Daily |
| Checking of Bearing of Motor Drive | Semi-annually |
| Renewal of Oil | Quarterly |

Bilge Oily Water Separator

| Items | Frequency |
| --- | --- |
| Checking and Cleaning of Bilge Suction | Weekly |
| Checking and Cleaning of Filter Element | Quarterly |
| Checking and Cleaning of 15ppm Alarm | Quarterly |
| Checking Casing and Pipes for Leakage | Daily |
| Checking of Valves and Cocks | Semi-Annually |

Others

| Items | Frequency |
| --- | --- |
| Piping Outside Machinery Spaces | Monthly |

  - Auxiliary Engines

| Items | Frequency |
| --- | --- |
| Exhaust Valves | Every 1,000 hours |
| Fuel Oil Valves | Every 480 hours |
| Starting Air Vales | Every 1,440 hours |
| Safety Valves | Every 1,440 hours |
| Cylinder Heads | Every 4,320 hours |
| Intake Valves | Every 2,880 hours |
| Pistons | Every 8,640 hours |
| Liners | Every 8,640 hours |
| Connecting Rod Bearing | Every 4,320 hours |
| Main Bearings | Every 8,640 hours |
| Outboard Bearings | Every 8,640 hours |
| Air Coolers (if any) | Every 1,440 hours |
| Fuel Oil Injection Pumps | Every 4,320 hours |
| Turbocharges (if any) | Every 4,320 hours |
| Air Filters (if any) | Every 72 hours |
| Fuel Oil Filters / Strainers | Every 144 hours |
| Lube Oil Filters / Strainers | Every 144 hours |
| Lube Oil Coolers | Every 360 hours |
| Freshwater Coolers (if fitted) | Every 480 hours |
| Fuel Oil Valve Cooler | Every 1,440 hours |
| Air Distributor Valve | Every 720 hours |

  - Electrical Machinery

Electrical Motors

| Items | Frequency |
| --- | --- |
| Checking of Megger Readings | Quarterly |
| Renewing of Bearings | Every 6-12 months or as needed |
| Overhauling: Open-up, Clean, Varnish and Close-up | If the megger reading is 1 mega-ohm or below, or as needed |

Generators

| Items | Frequency |
| --- | --- |
| Checking of Brushes and Slip-Ring (if any) | Monthly |
| Checking of Bearings | Quarterly |
| Checking of Lubrication of Bearings | Daily |
| Checking of Megger Readings | Quarterly |
| Overhauling: Open-up, Clean, Varnish and Close-up | If the megger reading is 1 mega-ohm or below, or as needed |

Switchboards

| Items | Frequency |
| --- | --- |
| Cleaning | Annually |
| Checking Lines and Contractors | Annually |
| Check Megger Readings | Annually |
| Checking of Bearings of Electrical Motors | Semi-annually |

  - Main Engines

| Items | Frequency |
| --- | --- |
| Liners | Every 8,640 hours |
| Cylinder Heads and Jacket Side | Every 2,160 hours |
| Intake Valves | Every 720 hours |
| Exhaust Valves | Every 720 hours |
| Fuel Oil Valves | Every 360 hours |
| Safety Valves | Every 4,320 hours |
| Main Bearings | Every 8,640 hours |
| Connected Rod Bearings | Every 8,640 hours |
| Outboard Bearings | Every 8,640 hours |
| Thrust Bearings | Every 8,640 hours |
| Air Starting Valves | Every 720 hours |
| Air Coolers | Every 2,160 hours |
| Fuel Oil Injection Pumps | Every 8,640 hours |
| Fuel Oil Filters / Strainers | Every 168 hours |
| Lube Oil Filters / Strainers | Every 168 hours |
| Air Suction Filters | Every 168 hour |
| Fresh Water Coolers | Every 1,440 hours |
| Lube Oil Coolers | Every 1,440 hours |
| Fuel Oil Valve Cooler | Every 1,440 hours |
| Reduction Lube Oil Coolers (if any) | Every 1,440 hours |
| Turbocharges | Every 4,320 hours |
| Air Distribution Valves | Every 360 hours |

ANNEX C

STANDBY EQUIPMENT TESTING INTERVALS

Emergency Steering

| Items | Frequency |
| --- | --- |
| Check Linkages of Looseness (if any) | Monthly |
| Check for Linkage – Hydraulic Oil | Monthly |
| Check (alternate) Power System (if any) | Monthly |
| Check Pins and Levers | Monthly |
| Check Shut – Off Valves (if any) | Monthly |
| Check Alternate Hydraulic Pump(s) (if any) | Monthly |
| Check Availability of Written Instruction Posted Within the Steering Compartment | Monthly |

Emergency Lighting

| Items | Frequency |
| --- | --- |
| Check Battery Charge | Monthly |
| Check Electric Bulb | Monthly |
| Check Charging Power Source | Monthly |
| Check Switch for Defects | Monthly |

Emergency Alarm

| Items | Frequency |
| --- | --- |
| Check Power Source | Monthly |
| Check Button / Switch | Monthly |
| Check Ringing Unit | Monthly |

Emergency Fire Pump

| Items | Frequency |
| --- | --- |
| Operational Test | Monthly |

Emergency Generators and Switchboards

| Items | Frequency |
| --- | --- |
| Operational Test | Monthly |

VHF Radios

| Items | Frequency |
| --- | --- |
| Check Battery Packs for Stored Power | Monthly |
| Check Battery Chargers | Monthly |
| Operational Test | Monthly |

ANNEX D

SEMI-ANNUAL ONBOARD DIAGNOSTIC TESTING

ANNEX E

LUBE OIL ANALYSIS

ANNEX F

WATER ANALYSIS

ANNEX G

VESSEL CRANE GENERAL REPAIRS AND MAINTENANCE

Mentioned below are the important points on general merchandise and inspection requirements that must be carried out in the vessel’s crane:

    - Daily Checks

Prior carrying out any operation using the vessel’s crane, a brief check on the following should be done:

- Check the lubrication

- Check the noise level by operating the crane without load

- Check the heat generation

- Check all the limits and trips are working properly

- Check the contact areas of electrical equipment

- Check the brake operation

- Check condition of clamp in the hook

    - Follow Proper Preventive Maintenance

The planned maintenance of crane should be followed properly and the following inspections should be done as per the working hours prescribed by the makers:

- Overhauling of motor

- Greasing of wires

- Renewal of wire ropes

- Annual survey

- Load test

    - Ensure Adequate Lubrication

Two lubricants are used:

- Grease: Wire ropes, rollers, plain bearings are applied with grease for smooth working.

Reduction gear for hoisting and sewing are not applied with grease. Check the greasing of these parts and replenish the grease every 2000 working hrs. or as prescribed by the makers. Ensure not to excessively apply the grease on moving parts as it may cause overheating at the time of rotation, which can have adverse effects.

- Oil: Lube oil is used for lubrication of ball bearing and roller bearing of hoisting and slewing gears.

Check the oil level regularly and replenish once the level is below the mark. The condition of oil needs to observed for stain, foam and dust. Renew the oil if the condition is not up to the requirement.

    - Check Minimum Requirements of Wire Ropes

The wire rope of a crane is the main operational part of the system, which is responsible to lift and move the load from one part of the engine room to another part. The following maintenance are to be performed on wire rope:

- Wind the wire rope in correct way to prevent kink of the rope

- Lubricate the wire rope at regular interval to reduce internal friction and to prevent corrosion

- Before greasing the rope, clean it with a cloth to remove dust and foreign matter

Requirements for the renewal of wire rope:

- When diameter of rope is reduced by 7%

- When the number of visible wire fracture is 15 over length of 6 times the wire rope diameter

- When the number of visible wire fracture is 30 over length of 60 times the wire rope diameter

- When a kink caused at some part of the rope does not restore to its original shape

- When the rope is reduced in length

- When the rope is twisted out of shape

- When the rope is deemed unusable because of rust or corrosion

    - Inspect the Gears

Gears are used to hoist or slewing the crane to maneuver the load at the desired location. The following inspection and maintenance are carried on the gear assembly:

- Drain the gear case oil if the condition is not good and after cleaning the case, replenish it with fresh lube oil

- Keep a check on the gear case noise while the crane is in operation

- Measure the thickness of the gear teeth and if has decreased by 20-25%, renew it with new one

- Inspect the gear for damage of teeth and renew it with new one if the damage is more

    - Check Condition of Sheave

Sheaves are used to carry the wire ropes from gearing chamber to the hook of the crane. Numbers of sheaves are installed in the assembly for the wire to move smoothly for operation. The following inspection must be carried out:

- Inspect the groove which is in contact with the wire for wear down

- Check all the sheaves for freedom of rotation

- Check the trueness of sheaves. It may bend if the load is lifted obliquely

- Check for cracks and broken circumference on the face of the sheave

- Check the bearings provided in the sheave assembly

- Check the bolt and the pin holding the sheave in place are in good condition

- Check each sheave to insure rope groove is smooth and free from burrs, or other surface defects

    - Carry Out Hook Maintenance

The hook assembly is such made that while the load is ON, it can rotate 360 degrees to adjust the position of the load and it comprises of bearings and drums. The following maintenance and checks needs to carried out:

- Lubrication of bearing to be done regularly

- If the partial wear of the part to be suspended with rope is not much worn, use grinder to make it an even surface

- Never repair any crack by welding

- Make sure the dowel pin holding the hook is in position. Measure the pin diameter and replace if its worn out

- Ensure safety latch spring is in working condition to avoid jumping of load

- If rotation lock is provided, check its operation

- Check the material for brittleness, cracking and hardness during annual survey

- If the part used for rigging the rope has worn out, check its shape (concave) and repair or replace as per the condition

- Check throat opening of the hook and if the throat opening exceeds 15% of the original opening, or if the hook is twisted more than 10 degrees, replace the same

    - Check The Brake

The engine room crane is equipped with electromagnetic brake with fail-safe arrangement. This is the most important safety arrangement provided in the crane. The following maintenance and inspection needs to be carried out:

- Measure the distance between magnet and armature. It should be approximately 0.5mm around the circumference

- If the gap is more or less than the recommended value, adjust the braking power nut to reach the required gap

- If any overhauling is carried out, supply the current tot the magnetic coil only to confirm that running shaft is operating without any resistance

- The electromagnetic brake is operated by the brake lining. Check for the moisture on the surface of the lining to avoid slippage

- Check for signs of overheating and mechanical damages

- Check the tension of spring which carries the brake during overhauling of the brake assembly

In addition to the abovementioned general repairs and maintenance guidelines, the following intervals are to be observed in the maintenance of vessel cranes:

| Items | Frequency |
| --- | --- |
| Main Transmitter and Receiver | Daily |
| Emergency Batteries | Weekly |
| Handheld Radios | Every Abandon Ship |
| VHF Radio | Daily |